Alternative Scenarios for Service Delivery

(track changes showing 2014 Update revisions to 2010 Plan)

|  | Scenario 1 | Scenario 2 | Scenario 3 |
| --- | --- | --- | --- |
| Description | Maximize asset life cycles | Moderate the decline of asset condition | Manage risk in a declining system |
| Description | Implements asset management approach, lifecycle costs are optimized, backlog is addressed, infrastructure condition is improved | Stabilizes system at current conditions in the short term and implements partial asset management approach; system continues to deteriorate over the long term | Available funding not adequate to maintain current condition of road network; continuing and accelerated decline leading to incremental shut down of the system; daily triage is the norm |
| Annual revenue needed | $330 million. $170–$180 million | $200 million. $120-$130 million | $110 million $102 million |
| Infrastructure Preservation Projects | Includes roadway subsurface, bridge, and pipe reconstruction on planned basis | Modest roadway and bridge replacement/ reconstruction to avoid accelerated future deterioration | Seal coat/overlay only; limited road, bridge, or drainage pipe replacement or reconstruction funded; deferred work creates escalating future cost liability |
| Capacity/system enhancements | None | None | None |
| Bridges (Inventory = 180) | - Improves current condition – keeps up with the replacement-rehabilitation backlog - 4 short span and 3 long span replacements per year | - Condition similar to current levels, but still continues to deteriorate over time deterioration is slowed over time - 2 short span and 2 long span replacements per year | - Eventual load limits, proactive load limiting to prevent damage, potential closures of “redundant” facilities - 1 short span replacement per year - No long span replacements |
| Drainage | - 16 fish culverts/year - Reduces other drainage backlog over 2/3 by 2024 | - 8 fish culverts/year - Reduces other drainage backlog over 1/3 by 2024 | - 4 fish culverts/year - Reduces other drainage backlog 1/4 by 2024 |
| Roadways Reconstruction – arterials only (Inventory = 450 miles) | Backlog down 25% by 2024 Improves current condition | Backlog down 5% by 2024 Condition similar to current levels in near term, but still continues to deteriorate over time; pavement condition and substructure slowly decline; some increase in localized flooding due to deferred maintenance of drainage infrastructure | No reconstruction Eventual speed reductions, lane closures for emergency repairs, proactive load-limiting to prevent damage, increased congestion, diminishing useful life of pavement overlays, closures of some “redundant” roads |
|  | Scenario 1 | Scenario 2 | Scenario 3 |
| Description | Maximize asset life cycles | Moderate the decline of asset condition | Manage risk in a declining system |
| Roadway Surface (Inventory = 1,500 miles) | Maintain weighted pavement condition score (WPCS) of 70 for arterial and local system | Maintain WPCS of 70 for arterials and 60 for locals | Maintain WPCS of 55 for arterials and 40 for locals |
| Maintenance facilities | Fully implement Facilities Master Plan (FMP) recommendations, including asset lifecycle management, repairs, functional upgrades, and long-term investment in rehabilitation or replacement | Implement most FMP recommendations, including repairs, functional upgrades, and long-term investment in rehabilitation or replacement | Address urgent, high-priority repairs and replacements identified in the FMP |
| Proactive vs. reactive | Allows cost-effective planned vs. reactive maintenance | Facilitates more cost effective planned vs. reactive maintenance; unscheduled repairs and associated temporary road closures will still be likely to occur | Reactive—little planned maintenance; maintenance needs/ costs accelerate as infrastructure condition deteriorates |
| Regulatory compliance | Met over time | Met over time | Met over time |
| Emergency response | High capacity. Response capability improved | Improved capacity. Staff and equipment are adequate to maintain current level of response | Limited capacity. emergency and storm response capability |
| Grant funding | Avoids loss of federal storm reimbursement and bridge grants | Avoids loss of federal storm reimbursement and bridge grants | Limited or lost |
| Mobility Improvements | Reduces backlog of intelligent transportation system (ITS) projects and non-motorized (pedestrian/bike) improvements by 2/3 by 2020 | Limited mobility improvements provided in conjunction with maintenance and preservation projects | May include limited grant-funded non-motorized improvements |
| Claims | Reduced | Stabilized | Escalate as risk increases |

The Pavement Condition Score (PCS) is a standard government pavement condition measure.  Scores correspond to the following categories:  50 – 100 good to excellent; 30-49 fair; 0 – 29 poor.