|  | Plan Review Matrix City of Enumclaw Comprehensive General Sewer Plan (September 2016) and Amendments 1 - 3 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Service Area The City of Enumclaw is located approximately 32 miles southeast of Seattle on a plateau between White and Green Rivers. The City has owned and operated a sewer collection system since the early 1900s. While the City's sewer system does not currently provide service to any properties in unincorporated King County, its service area does include properties in unincorporated King County. Sewer System The City's infrastructure includes approximately 48 miles of sewer lines and 11 lift stations. The City's existing treatment facility began operation in November 2008. Amendment #2 (August 2023) to the City's 2016 Comprehensive Sewer Plan updated its Sewer Capital Improvement Program for the 2024-2024 planning period. The amendment describes the projects the City has already completed, projects originally identified but not carried over, in addition to projects slated for updated planning period. For the updated planning period, 26 projects are identified with a total estimated cost of approximately $44,000,000. The 2016 Plan's Financial chapter indicated that at the time the City had six outstanding sewer debt issues which would be paid off by 2030. As noted in the Plan, this does not include new debt associated with the recommended CIP projects. The Plan does identify potential funding sources for the near-term projects, either "pay-as-you-go," utilizing revenue from monthly sewer rates and connection charges, or borrowing through a Washington state Department of Ecology Clean Water State Revolving Fund (SRF) loan. Reclaimed Water The Plan refers to a policy which states, the City will support the use of reclaimed water technologies where economically; however, it does not have any plans for wastewater reuse. SEPA The City completed a State Environmental Policy Act (SEPA) checklist and issued a SEPA determination for the 2016 Plan and each of the plan amendments. |  |  |  |
| # A review of the specific statutes, rules, codes, and policies to the water system plan is as follows: |  |  |  |  |
|  |  | A. General and sewer plan-specific requirements of King County Code (KCC) 13.24.010 | Comments/findings |  |
| (1) |  | Review and approval by the King County Council is applicable to public or private entities that provide sewer collection or treatment in unincorporated areas of King County as required by KCC 13.24 | - The City's sewer service area extends into unincorporated King County; however, it currently does not provide service to any properties in unincorporated King County. |  |
| (2) |  | The Plan shall be consistent with the King County Comprehensive Plan and development regulations and policies. | - The plan utilizes population and employment projections consistent with the City of Enumclaw's water system plan estimates which were determined using the King County Countywide Planning Policies (CPPs) Table DP-1 housing targets and Puget Sound Regional Council (PSRC) population projection data. - The Plan does reference restriction in providing sewer service in the rural area. |  |
| (3) |  | The Plan shall be adopted by that entity and approved by the King County Council as a prerequisite for the following: - Operating in unincorporated King County; - Approval of annexation proposals; - Granting of new right-of-way; franchises and right-of-way franchise renewals, and - Approval of right-of-way construction permits, except for emergency permits issued under KCC 14.44.055. | - 2016 Plan – The City approved the Plan on July 26, 2016 through Ordinance 2589. - Amendment #1 – The City approved the Plan amendment on November 25, 2019 through Ordinance 2668. - Amendment #2 – The City approved the Plan amendment on October 23, 2023 through Ordinance 2761. - Amendment #3 – The approved the Plan amendment on January 13, 2025 through Resolution 184. - The City has submitted a sewer franchise application to King County. |  |
| (4) |  | Plans should be submitted every six years or sooner if required by the Washington State Department of Ecology (DOE), or whenever sewer conditions have changed significantly within sewer service area. | - King County has not approved previous City sewer system plans. |  |
| (5) |  | Infrastructure for existing and future service areas based on adopted land use map. | - The plan utilizes population and employment projections consistent with the City of Enumclaw's water system plan estimates. The water system plan estimates were determined using the King County Countywide Planning Policies (CPPs) Table DP-1 housing targets and Puget Sound Regional Council (PSRC) population projection data. |  |
| (6) |  | Sufficient information to demonstrate the ability to provide service consistent with the requirements of all applicable statutes, codes, rules, and regulations. | - The Plan includes sufficient information to demonstrate the City can provide service consistent with applicable statues, codes, rules, and regulations. - DOE approved the 2016 plan on March 3, 2017, and amendment 2 on October 11, 2023, and amendment 2 on October 11, 2023. |  |
| (7) |  | Consistent with Washington Administrative Code (WAC) 173-240-050. | - DOE approved the 2016 plan on March 3, 2017, and amendment 2 on October 11, 2023. |  |
| (8) |  | Discuss the following: - Existing and planned flows, average and peak; - Existing and planned flows for any basin discharging into the County system; - Amounts of inflow and infiltration (I/I), in comparison with County standard of 1,100 gallons per-acre-per-day (gpad) and steps being taken to reduce; - Areas of concern regarding corrosion and odor control, and steps being taken, and - Opportunities for reclaimed water. | - The Plan does discuss existing and planned flows. - The Plan summarizes rain-induced infiltration and inflow. - The Plan indicates additional investigation is needed to verify the source of the I/I and then to develop a cost-effective rehabilitation program. - The Plan does address corrosion and odor control. - The Plan does state that the City supports reuse of water if economically feasible. |  |
|  |  | B. Public Sewer Service: KCC 13.24.035 |  |  |
| (9) |  | All developments within UGA served by sewer unless on-site systems are temporarily allowed per KCC 13.24.136 and 13.08.070. | - The Plan does reference the King County provisions requiring all developments within the UGA to be served by sewer unless on-site systems are temporarily. - The Plan does include a service area policy for properties within the City that requires connection unless specific conditions met and as interim service until service become available. |  |
| (10) |  | Required elements of a sewerage general plan, as called for in RCW 36.94.010(3), are included in King County Comprehensive Plan and Technical Appendix. | - The wastewater collected by the City's sewer system is treated by the City's sewer system. - The Plan has the general location and description of treatment and disposal facilities, and a general description of the collection system to serve the service area. - DOE approved the 2016 Plan on March 3, 2017, and amendment 2 on October 11, 2023. |  |
|  |  | C. Consistency requirements: KCC 13.24.060 |  |  |
| (11) |  | State and local health requirements. | - DOE approved the 2016 plan on March 3, 2017, and amendment 2 on October 11, 2023. - The Utilities Technical Review Committee (UTRC) review process included a representative of Public Health - Seattle & King County. |  |
| (12) |  | Elimination or prevention of duplicate facilities and a reduction of number of entities providing sewer service in King County. | - There are no neighboring sanitary sewer providers. |  |
| (13) |  | Promotion of most healthful and reliable services to the public. | - The Plan’s Operational policies and Capital Improvement Plan promotes healthful and reliable services to the utility’s customers. |  |
| (14) |  | Provision of service at a reasonable cost, and maximization of use of public facilities. | - The Plan includes an affordability analysis; the results indicate the City’s rates (in 2016) were considered affordable. |  |
| (15) |  | Basin wide or multibasin water plans, sewerage plans, or both when approved by DOE or Washington State Department of Health. | - The Plan does not refer to participation with basin wide or multi-basin water plans, or sewerage plans. |  |
| (16) |  | Applicable state water quality, water conservation (e.g., RCW 90.48.495), and waste management standards. | - Applicable state standards are met; DOE approved the 2016 plan on March 3, 2017, and amendment 2 on October 11, 2023 |  |
| (17) |  | Growth Management Act (GMA), chapter 36.70A RCW. | - The Plan conforms to the GMA. Please refer to policy consistency below. |  |
| (18) |  | Groundwater Management Plans. | - The Plan makes no reference to groundwater management plans. |  |
| (19) |  | Federally approved habitat conservation plans and recovery plans under Endangered Species Act. | - The Plan makes no reference to habitat conservation plans and recovery plans. |  |
| (20) |  | Requirements for salmon recovery under RCW 77.85, and other plans, including regional water supply or water resource management plans. | - The Plan makes no reference to the salmon recovery efforts within the watershed. |  |
| (21) |  | Applicable requirements to evaluate opportunities for the use of reclaimed water under chapter 90.46 RCW. | - The Plan indicates that the City will support the use of reclaimed water technologies where economically feasible. |  |
| (22) |  | State Environmental Policy Act documentation. | - 2016 plan – The City completed a SEPA checklist and subsequently issued a Determination of Nonsignificance (DNS) on March 16, 2016. There were no appeals. - Amendment #1 - The City completed a SEPA checklist and subsequently issued a DNS on September 18, 2019. An addendum was issued on October 16, 2019. There were no appeals. - Amendment #2 – The City completed a SEPA checklist and subsequently issued a DNS on September 6, 2023. There were no appeals. - Amendment #3 – The City completed a SEPA checklist and subsequently issued an Addendum to a DNS for the City's Draft Comprehensive Plan Periodic Update and Municipal Code Amendments on October 30, 2024. An appeal period is not required. |  |
|  |  | COUNTYWIDE PLANNING POLICIES |  |  |
| (23) |  | FW-4: Support fiscal sustainability for infrastructure and services that serve rural areas. | - The Plan does reference restriction in providing sewer service in the rural area. |  |
| (24) |  | PF-11: Require water reuse and reclamation where feasible. | - See #8 above. |  |
| (25) |  | DP-3 and PF-3: Coordination of capital facilities and service plans to promote efficient land use and reliable and cost-effective services. | - The review by the UTRC and approval by the County Council promotes coordination of capital facilities and service plans. |  |
| (26) |  | DP- 14: adopted infrastructure plans are consistent with adopted growth targets. | - The Plan utilizes population and employment projections consistent with the City of Enumclaw's water system plan estimates. The water system plan estimates were determined using the King County Countywide Planning Policies (CPPs) Table DP-1 housing targets and Puget Sound Regional Council (PSRC) population projection data. |  |
| (27) |  | DP-49, PF-1 and PF-13: Limit the extension of urban infrastructure improvements through the Rural Area. | - The Plan does reference restriction in providing sewer service in the rural area. |  |
| (28) |  | PF-12: With few exceptions, require development with the UGA to be served by public sewer. | - See #9 above. |  |
|  |  | KING COUNTY COMPREHENSIVE PLAN POLICIES |  |  |
| (29) |  | F-402: In the Rural and Natural Resource Lands, services provided by agencies shall support rural level of development and support service that meets the needs of the community and not facilitate urbanization. | - The sewer service area does not expand into the designated rural area. - The Plan does reference restriction in providing sewer service in the rural area. |  |
| (30) |  | F-406: The capital facility plans and capital improvement programs prepared by all other agencies that provide service to unincorporated areas should be consistent with the King County Comprehensive Plan. | - The General Sewer Plan is consistent with the GMA and King County Comprehensive Plan |  |
| (31) |  | F-444: All development within UGA to be served by public sewers, with some exceptions. | - See #9. Above. |  |
| (32) |  | F-445: King County and sewer utilities should jointly plan for phasing out of on-site systems within UGA. | - The Plan does not address jointly planning for connection properties currently utilizing an on-site septic system to the sewer system. - The Plan does include capital facility projects to identify potential sewer extension projects to service currently unsewered areas. The identified funding sources area developer extension projects and ULIDs. |  |
| (33) |  | F-447: The existing public sewer system in the Rural Towns of Vashon and Snoqualmie Pass shall not be expanded to serve land beyond the boundaries of the town, except as provided in Policy F-453 and as consistent with Title 57 RCW. | - Not Applicable |  |
| (34) |  | F-448: Sewer facilities such as pump stations, force mains, and trunk lines that do not provide connections to the Rural Area may be located in the Rural Area only when they are identified in a King County-approved comprehensive sewerage system plan an upon a finding by King County that it is technically necessary to provide service to the Urban Growth Area. | - The Plan does identify a pump station located in the Rural Area. The Plan further provides the technical justification for the location in the Rural Area. The UTRC, in its recommendation for approving the Plan, finds that the pump station is technically necessary to be located in the Rural Area. - The Plan does state that rural properties will not be connected to the City's sewer system. |  |
| (35) |  | F-453- Public sewer service shall be prohibited in the Rural Area or on Natural Resource Lands except where needed to address specific health and safety problems threating the use of existing structures, and the use of septic or other on-site wastewater systems has been determined by King County to be not feasible or to serve a new school authorized to be located in the Rural Area. An extension may only be approved if: a. it is tightlined, b. a finding by King County determining that no reasonable alternative technologies are feasible, and c. it does not add pressure to convert Rural land or Natural Resource Lands to urban area and densities or to expand nonresidential uses. | - Not Applicable |  |