# Review of the City of Snoqualmie General Sewer/Wastewater Facility Plan

Service Area The City of Snoqualmie owns and operates a wastewater system that serves the City and two small areas in unincorporated King County. The City provides wastewater service to customers throughout its corporate limits, the Echo Glen Children’s Center in unincorporated King County, the Snoqualmie Casino, and one parcel in the Potential Annexation Area. The City is located in the western foothills of the Cascade Mountains, approximately 25 miles east of Seattle. The City is along the Snoqualmie River and is surrounded by unincorporated King County except to the southeast where it abuts the City of North Bend. The City covers approximately 4,162 acres and has a population of approximately 11,039. The City owns and operates a municipal sewage collection and treatment system that includes 14 pump stations with associated force mains, approximately 43 miles of gravity sewer pipes and a water reclamation facility that is capable of producing reuse quality effluent and Class A biosolids. Reclaimed Water The water reclamation facility has one outfall to the Snoqualmie River immediately upstream of Snoqualmie Falls. Effluent is also pumped to a pond at the Snoqualmie Ridge Golf Course for reuse as irrigation water. - SEPA - The City completed a State Environmental Policy Act (SEPA) checklist for the Plan and, as lead agency under SEPA, issued a determination of nonsignificance for the approval of the Plan on February 11, 2013. There were no appeals.

|  | A. General, water and sewer plan specific King County Code (KCC) 13.24.010 | Comments/findings |
| --- | --- | --- |
| (1) | The code is applicable to water and or sewer utilities that obtain water in unincorporated areas, provide service in unincorporated areas of King County, and/or are component agencies of the regional wastewater system. | - The City collects wastewater in unincorporated areas of King County, making review under KCC 13.24 appropriate. - The City provides wastewater service to one parcel in the Potential Annexation Area and the Echo Glen Children’s Center. |
| (2) | Wastewater system plans should be submitted every six years or sooner if required by the Washington State Department of Ecology (DOE). | - The City’s last wastewater plan was completed in 2003. - The City will seek plan approval from DOE. |
| (3) | The infrastructure for existing and future service areas based on adopted land use map. | - Yes, King County land use and zoning maps were appropriately used. |
| (4) | The Plan contains sufficient information to demonstrate the ability to provide services consistent with the requirements of all applicable statutes, codes, rules, and regulations. | - Yes, the Plan demonstrates that the City has ample wastewater capacity to provide service consistent with the requirements of all applicable statutes, codes, rules, and regulations. |
| (5) | The Plan is consistent with Washington Administrative Code (WAC) 173-240 for the wastewater service. | - Yes, the plan is consistent with WAC 173-240-050. - The City is in the process of seeking plan approval from DOE. |
| (6) | Discuss the following: - Existing and planned flows average and peak; - Existing and planned flows for any basin discharging into county system; - Amounts of inflow and infiltration (I/I), in comparison with county standard of 1,100 gallons per-acre-per-day (gpad), and steps being taken to reduce; - Areas of concern regarding corrosion and odor control and steps being taken; - Opportunities for reclaimed water; and - Additional information required by the Utilities Technical Review Committee (UTRC). | - All are discussed in the Plan: - The existing and planned flows for average and peak are reasonable. - The City does not discharge into the County’s regional system. - Not applicable. - There were no issues identified with either corrosion or odor control. - The City reclaims water and uses it for irrigation. - Not applicable. |
|  | B. Public Sewer Service: 13.24.035 |  |
| (7) | All developments within Urban Growth Area (UGA) served by sewer unless on-site are allowed as temporary per KCC 13.24.136 and 13.08.070. | - Yes. Although the City’s service area is largely developed, there are unserved areas within the City’s sewer service area. |
| (8) | Public sewer provided in rural towns if approved by King County. | - There are no rural towns in the City’s sewer service area. |
| (9) | No public sewer service outside UGA unless authorized under KCC 13.24.134. | - The sewer service to the Echo Glen facility is tightlined and the City understands that no sewer connections are allowed to the line. - Within the Potential Annexation Area sewer service may be extended if and when existing onsite facilities fail. - Sewer service is consistent with KCC 13.24.035. |
| (10) | Required elements of sewerage general plan, as called for in RCW 36.94.010(3), are included in King County Comprehensive Plan and Technical Appendix. | - Yes, the Plan has the general location and description of treatment and disposal facilities, trunk and interceptor sewers, pumping stations, monitoring and control facilities, local service areas, and preliminary engineering in adequate detail to assure technical feasibility and, to the extent known, the methods of distributing the cost and expense of the system and the economic feasibility of plan implementation. |
|  | C. Consistency requirements: 13.24.060 |  |
| (11) | State and local health requirements. | - The Plan has not yet been reviewed and/or approved by DOE. - The UTRC review process included a representative of Public Health-Seattle and King County. |
| (12) | Elimination or prevention of duplicate facilities. | - Not applicable. |
| (13) | Promotion of most healthful and reliable services to the public. | - Yes. |
| (14) | Provision of service at a reasonable cost, and maximization of use of public facilities. | - Currently the City’s charges are affordable. - In general, the City collects revenues in excess of expenditures. |
| (15) | Reduction of number of entities providing sewer service in King County. | - The City has written agreements with other sewer providers regarding areas to be served in order to avoid overlapping jurisdiction and to ensure efficiency in the use of existing facilities. |
| (16) | Basin wide or multi-basin water plans, sewerage plans, or both when approved by DOE or Washington State Department of Health (DOH). | - The City is aware of, and supports, the planning done in the Snoqualmie basin for salmon recovery purposes. The City is not part of the regional wastewater system managed by King County. |
| (17) | Applicable state water quality, water conservation (e.g., RCW 90.48.495), and waste management standards. | - Yes. As the City implements more water conservation programs, there may be a minimal negative impact on the operation of the sewer system. |
| (18) | Growth Management Act (GMA) (RCW 36.70A). | - Yes, the Plan is consistent with the King County Comprehensive Plan and GMA. - The Plan is an element of the City’s Comprehensive Plan. |
| (19) | Groundwater Management Plans. | - The Plan does not reference the groundwater aquifer protection plan and how the sewer system impacts it. |
| (20) | Federally-approved habitat conservation plans and recovery plans under Endangered Species Act (ESA), and other plans, including regional water supply or water resource management plans. | - The Plan makes no reference to the salmon recovery efforts within the watershed. However, the City has participated in the salmon recovery effort for Water Resource Inventory Area 7. - The Plan is consistent with water resource management plans for the basin. |
| (21) | Applicable requirements to evaluate opportunities for the use of reclaimed water under chapter 90.46 RCW. | - Yes, the City supports using reclaimed water and seeks new use opportunities. |
| (22) | Sewer facilities allowed crossing rural areas only under certain conditions and only for schools or public health emergencies, under certain conditions. KCC 13.24.132. | - Not applicable. |
| (23) | State Environmental Policy Act documentation. | - Determination of nonsignificance issued by the City on February 11, 2013, with no appeals. |
|  | COUNTYWIDE PLANNING POLICIES |  |
| (24) | CA-5, CA-6, and E-466: adopt policies to protect the quantity and quality of groundwater, and develop strategies to compensate or mitigate for losses. | - Providing sewer service to properties without service consistent with the Plan may increase groundwater protection. |
| (25) | CO-7: water reuse and reclamation shall be encouraged, especially for high water users. | - See number 21. |
|  | KING COUNTY COMPREHENSIVE PLAN POLICIES |  |
| (26) | F-104: plan for provision of services to rural areas. | - No rural areas within the City’s service area. |
| (27) | E-105: protect critical habitat. | - Not applicable. |
| (28) | E-434: management and protection of water resources by King County through incentives, regulations, and programs. | - Not applicable. |
| (29) | E-477: protect and enhance surface waters, including Puget Sound. | - Not applicable. |
| (30) | F-105: work with cities and service providers to provide services. | - Yes, the City has agreements with neighboring service providers to ensure coordinated service. |
| (31) | F-202: ensure adequate supply of public facilities to support communities. | - Yes, the City included review and incorporation of information from the most recent King County Comprehensive Plan. |
| (32) | F-203: work with cities, special purpose utilities, and other service providers to define regional and local services and determine appropriate providers. | - Yes, the City has written agreements with local governments regarding service provision and coordination with other systems. |
| (33) | F-207: funding for growth should support facilities needed within UGAs, prioritized and coordinated through capital improvement programs (CIP), to comply with concurrency requirements. | - Funding sources are identified to support identified needs, including facilities to serve anticipated population growth under local comprehensive plans. - Extension of the sewer system will be primarily by developer extension agreements. |
| (34) | F-208: support rural levels of development and not facilitate urbanization. | - There are no rural areas within the City’s service area. |
| (35) | F-210: coordinate development of utility facilities. | - Yes, see number 32. |
| (36) | F-212: King County’s CIP demonstrates that projected needs for facilities and services can be met within the UGA in compliance with concurrency requirements; where not possible, identify strategies including phasing and financing. | - Yes, the City’s CIP identifies facilities and a funding strategy to ensure that it will meet anticipated demands. |
| (37) | F-213: water and sewer utilities that provide services to unincorporated King County shall prepare capital facility plans consistent with requirements of GMA and King County Comprehensive Plan. | - Yes. |
| (38) | F-215: King County to initiate sub-area planning process where any service provider declares, in its CIP, an inability to accommodate projected service needs within their service area. | - Not applicable. |
| (39) | F-217: where an area wide sewer, water, or transportation deficiency is identified, King County and applicable service providers shall remedy the deficiency through a joint planning process. | - Not applicable. |
| (40) | F-245: all development within UGA to be served by public sewers, with some limited exceptions. | - Yes, the City has policies in place to ensure and encourage the use of the sanitary sewer system within the urban area. - Extension of the sewer system will be primarily by developer extension agreements. |
| (41) | F-246: King County and sewer utilities should jointly plan for phasing out of on-site systems within UGA. | - The City acknowledges that some on-site systems exist within its service area and has policies in place to phase out their use over time. |
| (42) | F-249: public sewers may only be extended into rural and natural resource areas under limited conditions, only if they are tightlined, and only after specific findings are made; utilities shall have written agreements to ensure this; permitted public sewers shall not be allowed to convert rural or natural resource lands to urban uses and densities or to expand permitted nonresidential uses. | - Yes, the City understands and will abide with F-249. |
| (43) | F-250: facilities, such as pump stations, force mains, and trunk lines, which do not serve rural areas, may only be located in rural areas if they are identified in county-approved plans, and with a finding that such facilities are necessary to serve UGA. | - Yes, the City’s facilities are consistent with the policy. |
| (44) | F-251: on-site facilities in rural and natural resource areas shall be designed, constructed, and operated as permanent facilities. | - Not applicable. No rural or natural resource areas within the City. |
| (45) | F-252: King County should monitor failing on-site systems and analyze options which may include connecting to sewerage systems where consistent with the King County Comprehensive Plan. | - No significant number of failing on-site systems within the City’s service area of unincorporated King County. |
| (46) | F-253: collective on-site systems may only be used in rural and natural resource areas under specified conditions. | - Not applicable. No rural or natural resource areas within the City. |